Pumping module

CPU:

S7-300 314C-2 PN/DP

ASi:

PHOENIX CONTACT FLX ASI MA PB SF (2773597)

AS Interface (AS-i) Gateway for PROFIBUS DP with standard function,

IP20 degree of protection, as per AS-i specification 3.0

PHOENIX CONTACT ASI IO ME DIO 4/4 AB (2773542)

Digital input/output module, according to specification 3.0, degree of

protection IP20, 4 digital inputs, 24 V DC, 4 digital outputs, 24 V DC,

COMBICON plugs included

PHOENIX CONTACT ASI QUINT 100-240/2.4 EFD (2736686)

Power supply unit for AS interface, 2.4 A, integrated ground fault detector, IP20 degree of protection

DP/PA coupler:

PEPPERL-FUCHS SEGMENT COUPLER 1 (KFD2-BR-1.PA.93)

The segment coupler connects a PROFIBUS PA segment to a PROFIBUS segment based on RS 485 transfer technology in accordance with EN 50170. The Profibus is EN 50170/2 compliant. The communication process is transparent. The protocol is transferred without any changes. Because of this, the segment coupler works with any PROFIBUS protocol and no parameters are set for it. The baudrate of the PROFIBUS PA segment is 31.25 kbit/s in accordance with IEC 61158-2. The PROFIBUS segment based on the RS 485 transfer system has a baudrate of 93.75 kbit/s. This baudrate is especially suitable for mixed operation with DP and PROFIBUS PA stations. Power is supplied to devices on the PROFIBUS PA by the segment coupler through the bus.

Pressurized water module:

Distributed I/O, PHOENIX CONTACT:

TRIO-PS/ 1AC/24DC/ 5 (2866310)

IL PB BK DI8 DO4/EF-PAC (2692322)

IB IL AI 2-HART-PAC (2862149)

IB IL AI 8/SF-PAC (2861412)

or

IB IL AI 8/IS-PAC (2861661)

possibly also

IB IL 24 DO 2-PAC (2861470)

Another reference:

| Number | Product | Description |
| --- | --- | --- |
| 2708261 | PSI-MOS-PROFIB/FO 850 T |  |
| 2692322 | IL PB BK DI8 DO4/EF-PAC |  |
| 2861412 | IB IL AI 8/SF-PAC | 8*AI |
| 2862149 | IB IL AI 2-HART-PAC | 2*Hart AI |
| 2861250 | IB IL 24 DI 16-PAC | 16*DI |
| 2700172 | IB IL 24 DO8/HD-PAC | 8*DO |
| 2708245 | SUBCON-PLUS-PROFIB/PG/SC2 |  |
| 2313672 | SUBCON-PLUS-PROFIB/90/IDC |  |
| 2866763 | QUINT-PS/ 1AC/24DC/10 |  |

Cooling module:

Distributed I/O, WAGO:

PROFIBUS DP/V1 fieldbus coupler 12 mBaud digital and analog signals (750-333)

4-channel AI module 4-20 mA single ended (750-455) Note below.

2-channel AI module 4-20 mA single ended S.E. HART (750-482)

2-channel AO module 4-20 mA (750-554)

2-channel DI module DC 24 V 3,0 ms (750-400)

2-channel DO module DC 24 V 0,5 A (750-501)

End module (750-600)

Power source?

Note:

If differential channels are needed (instead of single-ended channels) in AI-module, product number is 750-454 (note! in this module only 2 channels)

“single ended”			      “differential”