PRIMARY TASK:

Make a control system (PLC configuration, program and control display) for the pumping module.

All actuators in the module have to be operable. All system inputs in the module have to be shown in control display. Control display has to be informative, clear, safe to use and user friendly in every way.

Application:

Study what kind of risks there is when operating this module. How you can prepare for those risks with your program? Make the interlocks needed to safe operation. One interlock is that always pump LFA01AP201 has to run if pump LFA01AP202 is running.

All measurements has to be scaled with one FC or FB and all the measurements has to be saved to one shared DB.

There is need only for one control loop. Control the flow at LFA01CF204 with speed of LFA01AP202. The program has to be done by using best practices. More detailed information about program specification and details when asked.

Project information:

There is many documents and other info in Moodle. If you need any other information, ask. Information in Moodle will be updated.

Main criteria for grade 1:

- Only manual operation possible

- Very basic control display

- No bus faults

- Some bad structures in program allowed

- Some defects in safety or operability allowed

- Some devices can be operated from control display and some measurements are showing values.

- Basic knowledge and project work skills, not independently managed

Main criteria for grade 2:

- There can be some minor malfunctions

- All devices can be operated from control displays and all measurements are showing right values.

- Basic knowledge, some difficulties in applying

Main criteria for grade 3:

- All the elements of good program and control, basic level

- Device specific FBs and FCs for scaling etc.

- Global DBs and instance DBs used (PLC memory used wisely)

- Everything works

- All automated functions are working, control loop ok

- Basic skills in engineer

Main criteria for grade 4:

- STL or SCL creative elements in the program

- Good sequence controls (all actuators has A and M mode)

- Good skills in engineering

Main criteria for grade 5:

- Storing register (DB) for one measurement (DB has to include 100 latest values (100 cycles) of control loop’s measurement). These values are used in counting an average of those values. That average is shown in control display.

- Some WinCC ActiveX tools used (trend, alarm etc.)

- Indirect addressing, UDT (PLC datatypes) and other more complex elements used wisely,

- Good control loop elements, tuned loop

- Functions build to help HMI operations (interactive and informative)

- Excellent skills in engineering. You can help others.